DK-AHD7066-5M

5mp Waterproof dome Camera

4 in 1 TVI/CVI/AHD/CVBS

Key Feature：

1. High preformance HD 5mp cmos Sensor

2. Day/Night(ICR), AWB, AGC, BLC,2D,3DDNR,

3. 24pcs infrared leds

4.Lens: 3.6mm/6mm/8mm/12mm optional

5.OSD function

6.Waterproof IP66

Picture:

Spec:

| Model | DK-AHD7066-5M |
| --- | --- |
| Image Sensor | 1/3" 5MP GC4653 cmos Sensor |
| Resolution | 5MP |
| Signal System | PAL/NTSC |
| Lens | 3.6/6/8mm/12mm fixed megapixel lens |
| Video Output | 5MP TVI/CVI/AHD/CVBS 4 in 1 |
| Led | 24pcs infrared，20m night vision |
| Function Control | OSD |
| Electronic Shutter | Auto/Manual |
| Minimum illumination | Color:0. 1Lux@(F1.2,AGC ON),0Lux with IR ON |
|  | Black and white: 0.01Lux@(F1.2,AGC ON),0Lux with IR |
| Languages Support | English, Chinese, Turkish, Dutch Portuguese, Spanish, Italian, French, German, Japanese, Korean |
| Day&Night | Yes, IR cut filter automatically switch |
| Shutter | 1/50(1/60)sec to 1/100,000sec |
| White balance | Auto/manual/ATW |
| White Balance | Auto |
| BLC/HLC | Auto |
| DWDR | Yes |
| 2DNR/3D NR | yes,both |
| Defog | Yes |
| Sens Up | Yes |
| Motion Detection | Up to 4 areas |
| Privacy Mask | Up to 4 area |
| S/N Ratio | >54dB(AGC off) |
| Advanced image | Sharpness |
| Mirror | Yes |
| Power supply | DC12V±10% |
| Working working Environment | (-10°C~+50°C),RH90% MAX(non-condencing) |
| Power Consumpution | 120mA~200mA |
| Transmission Distance | Over 500m via 75-3 coaxial cable |